}ULASER}.cw10

.rm6.5"

.ul on

.op

TO:       CONCERNED HAL STAFF

FROM:     A.B. White

SUBJECT:  ST-8000 VERSION 1.4F - RTS/CTS CONTROL

DATE:     April 24, 1990

.fo ABW  4-24-90  stmv14f.doc

To satisfy a customer requirement, we need to provide a fixed RTS to CTS delay

on the Terminal Data port of our ST-8000.  Even though we presently control

the CTS output on this port, the RTS input is not a control board processor

input.  To permit the processor to read the RTS input, a new jumper and new

firmware must be installed in the ST-8000.

1.  CONNECTING DATA TERMINAL RTS

1.1  Remove the DTR signal wire between Remote Control DB-25 (A5J7a) pin 20

and the 26 pin BERG connector pin 14.

1.2  Construct a new jumper wire to go from Terminal Data DB-25 (A1J9) pin 4

to the 26 pin BERG connector pin 14.  Install a BERG pin on one end

of this jumper, strip and tin the other end.

1.3  Solder the jumper to the Terminal Data DB-25 (A1J9) pin 4 on the

component side of the modem board.

1.4  Push the BERG pin end of the jumper into BERG pin 14.

2.  FIRMWARE V1.4F

2.1  Install ST-8000 V1.4F firmware at U10 and U11.

2.2  Turn ON the ST-8000; it should power up normally.

2.3  Connect an RS-232 breakout box to the Terminal Data port A1J9.

Turn PRINT SQUELCH fully counter clockwise to turn ON DCD.

2.4  Verify that the Terminal Data (A1J9) CTS output on pin 5 is OFF or LOW.

2.5  Connect the Terminal Data (A1J9) RTS input on pin 4 to the DCD output

on pin 8.  Verify that the Terminal Data (A1J9) CTS output on pin 5

turns ON or HIGH when the pins are connected.  NOTE: there is a 200ms

delay between RTS turning ON and CTS turning ON.

ST-8000  V1.4F  Checksum:  U10  1F00

U11  B300