.rm 65

.cw 12

.ul on

.op

.mt 4

.mb 7

.lh 8

.lq off

.ps off

.po 8

.sr 3

.cw 10

4.3  CONTROL ASSEMBLY A2

.fo ABW 9-14-90 ch4.doc

.he CHAPTER 4                Theory of Operation                            Page #

The  Control Board (Assembly A2) is a microprocessor based  sub-assembly  that

contains all Modem Board (Assembly A3) filter frequency synthesizers, the AFSK

transmit tone synthesizer, control bit latches, and the front panel  interface

circuits.   In  addition,  the  Transmit Data (TXD)  and  Receive  data  (RXD)

polarity,  regeneration, and receive clock recovery circuits are  included  on

this board.

The  following  sections contain detailed descriptions of  the  Control  Board

circuit  illustrated  in Figures A1806 to A1827.  Please refer  to  the  block

diagram in Figure 4.11 and the schematic diagrams during this discussion.

4.3.1 Microprocessor Controller

The  ST-8000A  Control  Board  (Assembly A2)  uses  a  conventional  dedicated

microprocessor  architecture  with 32K bytes of read only (EPROM)  memory  for

firmware  storage,  2K  bytes  of read/write (RAM) memory,  and  2K  bytes  of

electrically alterable (EEPROM) memory.  The Z80A microprocessor uses a 4.9152

MHz  clock (PCLK) generated by a TTL crystal oscillator.  The 330  ohm  pullup

resistor on U15-6 ensures that the clock signal meets the minimum high voltage

level requirements of the Z80A.  The Control Board uses both memory mapped and

I/O  port  mapped peripheral devices and latches.  The Z80A operates  with  no

wait states.

Note the 10K ohm pullup resistors on the microprocessor write, WRM_L (U15-22),

and read, RDM_L (U15-21), control signals.  These resistors hold these control

lines  high when the processor is in the power on reset condition since  these

control  lines  are  in a high impedance tri-state  condition  at  that  time.

Without  the pullup resistors, a false memory write signal might  corrupt  the

parameters stored in the non-volatile parameter storage memory (U2).

The  Z80A  has two maskable interrupt sources: the serial  channel  controller

interrupt,  INT_L (U56-12), and the front panel keypad interrupt, KBINT  (U64-

1).  These two interrupt sources are combined by a 2 input NAND gate (U56)  to

produce a common interrupt signal to drive the microprocessor interrupt  input

(U15-16).  The front panel interrupt signal, KBINT, must be inverted since  it

is  an active high signal.  When an interrupt occurs, the microprocessor  must

poll  both  the serial controller (U66) shown in Figure A1815  and  the  front

panel controller to see which device requested the interrupt.

The non-maskable interrupt, NMI (U15-17), provides a real time clock reference

for  correctly  calculating ST-8000A timing delays.  This square  wave  timing

signal is generated by the programmable timer (U1) in Figure A1807, and it  is

set for a 0.5 ms period.

The  RESET  (U9-5)  and  RESET_L (U9-6, U15-26) signals  are  generated  by  a

"deadman"  timer  and  power on reset circuit (U9).   During  normal  ST-8000A

operation,  the  deadman timer reset signal, SANITY_L (U9-7),  is  pulsed  low

periodically by the microprocessor.  Should some abnormal situation cause  the

microprocessor  to  cease operation, the deadman timer will force  a  hardware

reset of the ST-8000A.  When a reset occurs, the RESET and RESET_L outputs are

pulsed low and high, respectively, for approximately 0.25 seconds.

Due  to the number of latches and devices on the control board, the Z80A  read

and  write control signals are buffered by two buffer gates to  generate  RD_L

(U10-8)  and WR_L (U10-11), respectively.  Note that the  un-buffered  signals

RDM_L  and WRM_L are connected directly to the EPROM (U4), the RAM  (U3),  and

the EEPROM (U2).

4.3.2  System Clocks

A  programmable timer (U1) in Figure A1807 generates the  reference  frequency

for  all  of the frequency synthesizers, CKREF (U1-10), the  low  pass  filter

clock,  LPCLK (U1-13), and the microprocessor real time reference  clock,  NMI

(U1-17).

All  of the ST-8000A frequency synthesizers are referenced to the same  4.9152

MHz  crystal oscillator that drives the microprocessor clock, PCLK  (U1-9,15).

One  16  bit programmable timer divides PCLK by 6 to generate the  819,200  Hz

reference clock, CKREF (U1-10), used by all filter synthesizers.  In addition,

the 4.9152 MHz clock is divided by various constants to generate the low  pass

filter  clock,  LPCLK (U1-13), for the modem board.  An  inverting  buffer  is

provided on CKREF to increase the fan-out capability.

A  separate  6.144 MHz oscillator reference is used for the NMI  clock.   This

clock is divided by 3072 to produce a 2000 Hz square wave signal, NMI (U1-17),

for  the 0.5 ms real time clock.  Note that the NMI signal is  disabled  until

the NMI control signal, NMIENA (U1-16) is set high.

The  block  diagram in Figure 4.12 identifies the  significant  Control  Board

(Assembly A2) clock signals.

4.3.3 Memory and I/O Address Map

The  microprocessor memory is divided into memory mapped and I/O  port  mapped

sections.  All memory and I/O port address decoding is illustrated in  Figures

A1806 and A1807.

Decoding for the firmware memory (U4) is illustrated in Figure A1806.   Memory

address bit 15, A15 (U15-5), and memory request, MREQ_L (U15-19) are  combined

by a two input OR gate (U10) to generate the EPROM chip select signal  (U4-20)

when both signals are low.

Except  for  the  firmware  memory chip select, all  memory  and  I/O  address

decoding  is  contained in Figure A1807.  A 3-to-8 line decoder (U11)  is  the

primary memory mapped device decoder, and it is enabled by the combination  of

address bit 15, A15 (U11-6), high and memory request, MREQ_L (U11-5), low.   A

second  address decoder (U12) provides additional decoding for  several  write

only  latches  when it is enabled by write, WR_L (U12-4), low and  one  memory

address select line (U11-13) low.

The  I/O  port mapped devices are decoded by two 3-to-8  line  decoders  (U13,

U14).   The devices on the first decoder (U13) are both I/O input  and  output

peripheral devices while those on the second decoder (U14) are I/O output only

latched registers.

The  memory and I/O address map for the ST-8000A control board  is  summarized

below (all addresses are shown in HEX format):

MEMORY MAPPED ADDRESSES

ADDRESS         DESCRIPTION                                    R/W

--------------  ---------------------------------------------  ----------

0000H - 7FFFH   U4-20    27256 EPROM for firmware storage      READ

8000H - 87FFH   U3-18    2K x 8 RAM for temporary storage      READ/WRITE

8800H - 8FFFH   U2-18    2K x 8 EEPROM for parameter storage   READ/WRITE

A000H - A007H   U12-5    Synthesizer latches                   WRITE

A000H           U41-11   MARK transmit tone, low byte         WRITE

A001H           U43-11   MARK transmit tone, high byte        WRITE

A002H           U40-11   SPACE transmit tone, low byte        WRITE

A003H           U42-11   SPACE transmit tone, high byte       WRITE

A004H           U45-11   MARK filter synthesizer, low byte    WRITE

A005H           U46-11   MARK filter synthesizer, high byte   WRITE

A006H           U49-11   SPACE filter synthesizer, low byte   WRITE

A007H           U50-11   SPACE filter synthesizer, high byte  WRITE

E000H           U5-1,19  Transmit delay switch SW1             READ

E800H           U6-1,19  Transmit delay switch SW2             READ

I/O PORT MAPPED ADDRESSES

PORT       DESCRIPTION                                      I/O

---------  -----------------------------------------------  ----------

00H        U66-33    Serial controller                      INPUT/OUTPUT

08H

10H        J8-12     Front Panel select (FP_L)              INPUT/OUTPUT

18H        U1-21     Programmable timer                     INPUT/OUTPUT

20H        U8-1,19   Unit Address switch SW4                INPUT

28H        U7-1,19   Remote Port rate switch SW3            INPUT

30H        U44-1,19  AMHO, level, RS232/MIL, KBINT, TXD     INPUT

38H - 3FH  U14-5     Misc. control latches                  OUTPUT

38H        U47-11    MARK and SPACE filter Q latch         OUTPUT

39H        U48-11    NMI, Regen, RX/TX control latch       OUTPUT

3AH        U51-11    AMHC, Det. Mode, Channel, Mute, BIT   OUTPUT

3BH        U9-7      Sanity timer                          OUTPUT

3CH        J8-13     Front panel LED latch 0 (LED0_L)      OUTPUT

3DH        J8-14     Front panel LED latch 1 (LED1_L)      OUTPUT

3EH        U52-11    High speed RX synthesizer, low byte   OUTPUT

3FH        U53-11    High speed RX synthesizer, high byte  OUTPUT

4.3.4  EPROM, EEPROM, and RAM Memory

The ST-8000A memory address map is summarized below:

MEMORY ADDRESSES

ADDRESS         DESCRIPTION                                    R/W

--------------  ---------------------------------------------  ----------

0000H - 7FFFH   U4-20    27256 EPROM for firmware storage      READ 8000H - 87FFH   U3-18    2K x 8 RAM for temporary storage      READ/WRITE

8800H - 8FFFH   U2-18    2K x 8 EEPROM for parameter storage   READ/WRITE

Figure  A1806 illustrates the firmware EPROM (U4), the non-volatile  parameter

EEPROM (U2), and the 2k x 8 RAM memory (U3).  The ST-8000A firmware is  loaded

in  an industry standard 32k x 8 byte EPROM with 250 ns access time.  In  this

unit, address bit 15, A15 (U15-5) selects either the EPROM when A15 is low  or

the  EEPROM  and RAM when A15 is high.  Note that the memory  request  signal,

MREQ_L (U15-19) enables memory decoding only during memory access cycles,  not

during  I/O port access cycles.  The microprocessor read signal,  RDM_L  (U15-

21), is connected directly to the EPROM (U4-22), RAM (U3-20), and EEPROM  (U2-

20) output enable inputs.  The microprocessor write signal, WRM_L (U15-22), is

connected to the RAM write input (U3-21) and the EEPROM write input (U2-21).

4.3.5  Memory Mapped Latches and Buffers

The  ST-8000A  controls the MARK and SPACE transmit tone and  bandpass  filter

synthesizers  through memory mapped latches.  In addition, the transmit  delay

time  switch setting is read from memory mapped switch buffers.  These  buffer

addresses are summarized below:

MEMORY MAPPED LATCHES AND BUFFERS

ADDRESS         DESCRIPTION                                    R/W

--------------  ---------------------------------------------  ----------

A000H - A007H   U12-5    Synthesizer latches                   WRITE

A000H           U41-11   MARK transmit tone, low byte         WRITE

A001H           U43-11   MARK transmit tone, high byte        WRITE

A002H           U40-11   SPACE transmit tone, low byte        WRITE

A003H           U42-11   SPACE transmit tone, high byte       WRITE

A004H           U45-11   MARK filter synthesizer, low byte    WRITE

A005H           U46-11   MARK filter synthesizer, high byte   WRITE

A006H           U49-11   SPACE filter synthesizer, low byte   WRITE

A007H           U50-11   SPACE filter synthesizer, high byte  WRITE

E000H           U5-1,19  Transmit delay switch SW1             READ

E800H           U6-1,19  Transmit delay switch SW2             READ

Each  of the synthesizers requires a 16 bit constant to set the output  square

wave  frequency.  A detailed description of these registers is presented in  a

subsequent  section and only the memory addresses are listed here.

Two  other modem square wave signals are generated on the control  board;  the

high  speed synthesizer, HSCLK (U23-19), and the low pass filter clock,  LPCLK

(U64-12).   The  HSCLK synthesizer uses I/O mapped latches while  LPCLK  is  a

square wave signal generated by the on board programmable timer (U1).

The  ST-8000A  reads  the  transmit delay setting as a 16  bit  value  set  by

switches  SW1  and SW2 in Figure A1810.  For both switches, the  OPEN  or  OFF

position is read as a TTL 1 or high signal while CLOSED or ON is read as TTL 0

or low signal.

4.3.6  I/O Mapped Peripherals

The  ST-8000A  uses  I/O input and output  addresses  for  several  peripheral

devices and control bit latches.  An I/O memory map is shown below:

I/O PORT MAPPED ADDRESSES

PORT       DESCRIPTION                                      I/O

---------  -----------------------------------------------  ----------

00H        U66-33    Serial controller - Regen control      INPUT/OUTPUT

01H        U66-33    Serial controller - Regen data         INPUT/OUTPUT

02H        U66-33    Serial controller - Remote control     INPUT/OUTPUT

03H        U66-33    Serial controller - Remote data        INPUT/OUTPUT

10H        J8-12     Front Panel controller - data          INPUT/OUTPUT

11H        J8-12     Front Panel controller - control       INPUT/OUTPUT

18H        U1-21     Programmable timer - CKREF clock       INPUT/OUTPUT

19H        U1-21     Programmable timer - LPCLK clock       INPUT/OUTPUT

1AH        U1-21     Programmable timer - NMI clock         INPUT/OUTPUT

1BH        U1-21     Programmable timer - control register  INPUT/OUTPUT

20H        U8-1,19   Unit Address switch SW4                INPUT

28H        U7-1,19   Remote Port rate switch SW3            INPUT

30H        U44-1,19  AMHO, level, RS232/MIL, KBINT, TXD     INPUT

38H        U47-11    MARK and SPACE filter Q latch          OUTPUT

39H        U48-11    NMI, Regen, RX/TX control latch        OUTPUT

3AH        U51-11    AMHC, Det. Mode, Channel, Mute, BIT    OUTPUT

3BH        U9-7      Sanity timer                           OUTPUT

3CH        J8-13     Front panel LED latch 0 (LED0_L)       OUTPUT

3DH        J8-14     Front panel LED latch 1 (LED1_L)       OUTPUT

3EH        U52-11    High speed RX synthesizer, low byte    OUTPUT

3FH        U53-11    High speed RX synthesizer, high byte   OUTPUT

Figure  A1815  illustrates the dual channel serial controller (U66)  used  for

receive data regeneration and the remote control port.  This serial controller

contains internal baud rate generators to independently set the data rate  for

the  remote  control  port  and receive data  regeneration.   For  a  detailed

description  of  the  regeneration circuit and  remote  control  circuit,  see

Section x.x.x.

The front panel controller is a programmable keyboard/display interface device

described  in  a  subsequent  section.  For this  discussion,  note  that  the

interface requires two I/O addresses; one for control and the other for data.

The  programmable  timer,  U1, in Figure A1807, contains  three  16  dividers.

These  registers  are  loaded  at the I/O addresses shown  above  to  set  the

selected  low pass filter cutoff frequency, LPCLK (U1-13), and the  two  fixed

rate clocks: NMI_L (U1-01) and CKREF (U64-12).

The  remaining  I/O address assignments control  various  operational  latched

output bits.  The sections below detail each of these registers.

4.3.6.1  UNIT ADDRESS and REMOTE PORT RATE

PORT       DESCRIPTION                                      I/O

---------  -----------------------------------------------  ----------

20H        U8-1,19   Unit Address switch SW4                INPUT

28H        U7-1,19   Remote Port rate switch SW3            INPUT

The Unit Address switch, SW4 in Figure A1809, sets the ST-8000A unit  address.

This register is an 8 bit, read only input.

The  remote port rate switch, SW3 in Figure A1809, selects the data  rate  for

the remote control port.  This registers is an 8 bit, read only input.

4.3.6.2  RECEIVE SIGNAL LEVEL and STATUS BITS

PORT       DESCRIPTION                                      I/O

---------  -----------------------------------------------  ----------

30H        U44-1,19  AMHO, level, RS232/MIL, KBINT, TXD     INPUT

The  microprocessor  can read the audio level of the receive  signal  and  the

Automatic Mark Hold condition with an I/O mapped buffer shown in Figure A1808.

In   addition,  this  register  contains  the  state  of  the  transmit   data

RS232/MIL188  option  jumper  setting in Figure A1816,  and  the  remote  port

RS232/MIL188 option jumper setting in Figure A1815.

The state of KBINT (U64-2) in Figure A1806 is also provided in this  register.

Since two different interrupt sources are combined in a single  microprocessor

interrupt signal, this register may be read to see whether the interrupt  came

from the front panel keyboard/display controller.

The state of the TXD input may be read in this register as well.

The bit assignments in this register are shown below:

Input: LEVEL_L

BIT  PIN     DESCRIPTION

---  ------  --------------------------------------------------------------

0   U44-4   J7-23  Automatic Mark Hold (AMHO):  1 = LOS, 0 = SIGNAL

1   U44-17  J7-20  AGC Gain A (DGA)

2   U44-2   J7-21  AGC Gain B (DGB)

3   U44-6   J7-22  AGC Gain C (DGC)

4   U44-11  U64-2  Front Panel controller Interrupt (KBINT'): 0 = intr req

5   U44-15  J2SW   Transmit Data RS232/MIL188: 1 = MIL188, 0 = RS232

6   U44-8   J5SW   Remote Port RS232/MIL188:   1 = MIL188, 0 = RS232

7   U44-13  U59-3  Transmit Data input (TXD'): 1 = MARK, 0 = SPACE

4.3.6.3  MARK/SPACE BANDPASS FILTER Q

PORT       DESCRIPTION                                      I/O

---------  -----------------------------------------------  ----------

38H        U47-11    MARK and SPACE filter Q latch          OUTPUT

The  Q's of the MARK and SPACE input filters on the modem board  are  selected

with latched control bits on the Control Board.  Since each filter requires  4

bits, a single 8 bit latch is used as shown in Figure A1808.

Output: OUT0_L

PIN       DESCRIPTION

--------  -----------------------------------------

U47-2     MARK Filter Q High/Low           to J7-15

U47-5     MARK Filter Q bit 3              to J7-14

U47-6     MARK Filter Q bit 2              to J7-13

U47-9     MARK Filter Q bit 1              to J7-12

U47-12    SPACE Filter Q High/Low          to J7-19

U47-15    SPACE Filter Q bit 3             to J7-18

U47-16    SPACE Filter Q bit 2             to J7-17

U47-19    SPACE Filter Q bit 1             to J7-16

The Q values selected with these control bits are listed in Table 4.2.

4.3.6.4  CONTROL REGISTER 1

PORT       DESCRIPTION                                      I/O

---------  -----------------------------------------------  ----------

39H        U48-11    NMI, Regen, RX/TX control latch        OUTPUT

Several  miscellaneous  control signals are latched in an octal  D-type  latch

(U48) shown in Figure A1808.  This latch contains a reset input, RESET_L (U48-

1),  that will force all of the control signals to a low state when the  power

is  first turned ON.  This reset ensures that the NMI control  signal,  NMIENA

(U48-2), signal is low during power on initialization.

The function of each control signal is summarized below:

Output: OUT1_L, BIT 7

PIN       DESCRIPTION

--------  -----------------------------------------

U48-2     NMI Enable bit (NMIENA)          to U1-16

The NMI Enable bit (NMIENA) activates the NMI real time clock in Figure  A1807

for the microprocessor.  A high signal enables NMI (NMIENA = 1).  When a reset

occurs, this bit is forced low to disable NMI (NMIENA = 0).

Output: OUT1_L, BIT 6

PIN       DESCRIPTION

--------  ------------------------------------------------ U48-5     REGEN Enable (REGEN)             to U63-5, U64-5

The  Regeneration Enable bit (REGEN) determines whether the data terminal  RXD

output in Figure A1816 is connected to the demodulated RX data from the  modem

or  to the control board regeneration serial controller.   When   regeneration

mode is disabled (REGEN = 0), the modem receive data signal, RXDTR (U57-2), is

connected  to  the modem terminal receive data output, RXDFR  (U63-11).   When

regeneration  is  enabled  (REGEN  = 1) the  modem  receive  data,  RXDTR,  is

connected to the regeneration serial controller receive data input, RXDB (U66-

27)  and the regenerated data from the transmit data output, TXDB (U66-25)  is

connected  to  the modem terminal data connector receive data  output,  RXDFR.

For regeneration to work properly, the correct data rate and word length  must

be programmed into the serial controller channel B.

Output: OUT1_L, BIT 5

PIN       DESCRIPTION

--------  --------------------------------------------------------

U48-6     Resync RXD (RSYNC)               to U64-11, U56-1, U65-2

The ST-8000A has the ability to resynchronize the receive data, RXDTR,  signal

to  a recovered receive data clock.  D-latch U55 in Figure A1816  samples  the

RXDTR  signal from the modem  and re-clocks this data with a  stable  internal

data  rate clock.  The result of this resynchronizing operation is  to  reduce

the  jitter  in the data terminal receive data output, RXDFR  (U63-11).   When

enabled (RSYNC = 1) the receive data is resynchronized.  When disabled  (RSYNC

= 0) receive data is passed to the data terminal without re-clocking.

Output: OUT1_L, BIT 4

PIN       DESCRIPTION

--------  -----------------------------------------

U48-9     RXD Normal/Reverse  (RXDNR)      to U57-1

The  ST-8000A  provides  independent control over transmit  and  receive  data

polarity, as shown in Figure A1816.  When set for NORMAL receive data (RXDNR =

0),  RXDTR  from the modem is passed through to RXDB (U57-3)  without  change.

When enabled (RXDNR = 1) the RXDTR signal from the modem is inverted before it

reaches RXDB (U57-3).

Output: OUT1_L, BIT 3

PIN       DESCRIPTION

--------  -----------------------------------------

U48-12    TXD Normal/Reverse  (TXDNR)      to U57-4

The  ST-8000A  provides  independent control over transmit  and  receive  data

polarity, as shown in Figure A1816.  When set for NORMAL transmit data  (TXDNR

=  0), TXDIN from the modem is passed through to TXD (U57-6)  without  change.

When enabled (TXDNR = 1) the TXDIN signal from the modem is inverted before it

reaches TXD.

Output: OUT1_L, BIT 2

PIN       DESCRIPTION

--------  -----------------------------------------

U48-15    TXD Enable (TXENABLE)            to U56-10

The  ST-8000A  Built  In Tests (BIT) require that the external  TXD  input  be

disabled during testing.  When disabled (TXENABLE = 0) the data terminal TXDIN

input  is  blocked at U56-8.  When enabled (TXENABLE = 1)  the  data  terminal

TXDIN input is connected to the TXD (U57-6) output.

Output: OUT1_L, BIT 1

PIN       DESCRIPTION

--------  -----------------------------------------

U48-16    [not assigned]

Output: OUT1_L, BIT 0

PIN       DESCRIPTION

--------  -----------------------------------------

U48-19    Diversity Enable  (DIVC)         to J7-25

If  the  diversity  option is installed in the  ST-8000A,  diversity  mode  is

enabled when this signal is high (DIVC = 1) and disabled otherwise.

4.3.6.5  CONTROL REGISTER 2

PORT       DESCRIPTION                                      I/O

---------  -----------------------------------------------  ----------

3AH        U51-11    AMHC, Det. Mode, Channel, Mute, BIT    OUTPUT

Several  miscellaneous  control signals are latched in an octal  D-type  latch

(U51)  shown in Figure A1808.  All of these control signals are passed to  the

modem board.

The function of each control signal is summarized below:

Output: OUT2_L, BIT 7

PIN       DESCRIPTION

--------  ------------------------------------------

U51-2     Automatic Mark Hold Enable (AMHC) to J7-24

The  Automatic  Mark Hold Enable signal (AMHC) enables and  disables  the  AMH

circuit  on the modem board.  The automatic mark hold feature is enabled  when

AMHC = 1 and is disabled when AMHC = 0.

Output: OUT2_L, BIT 6

PIN       DESCRIPTION

--------  -----------------------------------------

U51-5     High Speed Enable (HSLS)         to J7-26

The  High Speed Enable selects the low or high speed demodulator on the  modem

card.   The  high speed demodulator is enabled when HSLS = 1 and  is  disabled

when HSLS = 0.

Output: OUT2_L, BIT 5,4

PIN       DESCRIPTION

--------  -----------------------------------------

U51-6     Detector Mode (DMA)              to J7-27

U51-9     Detector Mode (DMB)              to J7-28

The  detector mode is selected with these two bits.  The detector options  are

listed below:

B  A   Detector Mode

-  -   -------------

0  0   MARK/SPACE

0  1   MARK ONLY

1  0   SPACE ONLY

1  1   MARK/SPACE = ATC.

Output: OUT2_L, BIT 3

PIN       DESCRIPTION

--------  -----------------------------------------

U51-12    Channel Select (CHSEL)           to J7-32

The  modem  board has two channels: the modulator and  the  demodulator.   The

Channel Select control signal determines whether the transmit or receive tones

are  displayed  on  the front panel bar graph displays.   The  demodulator  is

selected  when CHSEL = 1, and the modulator channel is selected when CHSEL   =

0.

Output: OUT2_L, BIT 2

PIN       DESCRIPTION

--------  -----------------------------------------

U51-15    Transmit Mute (MUTE)             to J7-33

The  Transmit  Mute signal enables and disables the output AFSK  tones.   When

MUTE  =  0,  the transmit tones are enabled and the  radio  keyline  relay  is

activated or closed.  When MUTE = 1, the transmit tones are disabled, and  the

PTT relay is not activated.

Output: OUT2_L, BIT 1

PIN       DESCRIPTION                       --------  -----------------------------------------

U51-16    Loopback Enable (BCB)            to J7-36

U51-19    Loopback Enable (BCA)            to J7-34

The  Loopback enable control bits enable and disable an analog  loopback  that

connects the AFSK output signal to the audio input.  Three different  loopback

gain options are available:

B  A   Loopback Gain

-  -   ----------------

0  0   Disabled

0  1   0 dBm

1  0   -20 dBm

1  1   -40 dBm

4.3.7  MARK/SPACE Transmit Tone Generator

The  AFSK tone oscillator illustrated in Figure A1811 is a 16 bit  full  adder

operating as a digital signal synthesizer with a square wave output set to  50

times  the audio tone frequency.  Two sets of latches store the constants  for

the MARK and SPACE tones,  and the transmit data signal, TXD (U55-12), selects

either the MARK tone (TXD = 0) or SPACE tone (TXD = 1).

The D-type latch is included to synchronize the changes between MARK and SPACE

to that portion of the synthesized sine wave output where the slope is 0.   In

this  fashion, the distortion is minimized.  The synchronizing signal,  FSKCLK

(U55-11) pulses once per sine wave cycle.

The  MARK/SPACE  tone digital synthesizer is a 16 bit full  adder  implemented

with four 4 bit full adders, U24 - U27.  On each rising edge of CKREF (U17-11,

U16-11), a 819,000 Hz clock, the current 16 bit sum is latched into two D-type

latches, (U16, U17).  These latch outputs are then added to the tone  constant

stored  in either the MARK or SPACE tone latch depending on the state  of  the

sampled  TXD  signal  (U55-9, U55-8).  When a MARK is  transmitted,  the  MARK

constant  registers (U41, U43) are enabled on the adder inputs.  When a  SPACE

is  transmitted,  the SPACE constant registers (U40, U42) are enabled  on  the

adder  inputs.  The selected constant is summed with the previous sum  latched

in U16 and U17 to form a new sum that will be latched on the next rising  edge

of CKREF to complete the cycle.  The output of this synthesizer, TFSK (U17-19)

has a nominal frequency of 50 times the selected tone frequency.

Figure  4.13  summarizes  the digital frequency  synthesizer  operation.   The

digital  synthesizers generate their output frequency through  addition.   The

frequency is determined by the magnitude of the constant loaded into the input

latches  and  the  clock reference frequency  driving  the  synthesizer.   For

example,  if  a constant 001 hex is loaded into the MARK latch, and TXD  =  0,

then the sum at the output of the latches U16 and U17 will be incremented by 1

count  for  each rising edge of CKREF.  After 32,768 rising  edges,  the  TFSK

output will change from 0 to 1.  Then, after another 32,768 rising edges,  the

TFSK  signal will change from 1 to 0.  The total time for one cycle is  65,536

clock  cycles of CKREF.  If CKREF is set to 819,200 Hz, the TFSK  output  will

have  a frequency of 12.5 Hz.  If the MARK latch is changed to a  constant  of

002 hex, then TFSK will equal 25 Hz.  The largest constant that can be  loaded

into  the  MARK  register  is  32,767, and  the  resulting  output  signal  is approximately 409,587 Hz.  Thus, this synthesizer may be set to any  frequency

between 12.5 Hz and 409,600 Hz in steps of 12.5 Hz.

At  low frequencies or when the constant is an exact divisor of  819,200,  the

output  waveform  on  TFSK (U17-19) is nearly a 50% duty  cycle  square  wave.

However,  if the constant is not an integer divisor, the relationship  between

the constant and CKREF generates signal jitter.  While this jitter is  visible

on  an  oscilloscope, the resulting sine wave output is smoothed by  the  sine

wave approximation process and the 50 times over sampling.

In  the ST-8000A the tone synthesizer generates output frequencies on TFSK  of

15,000  Hz  and 150,000 Hz in steps of 25 Hz.  The resulting output  from  the

AFSK generator on the modem card is 300 to 3000 Hz in 0.5 Hz increments.   For

each  tone frequency, the constant loaded into the MARK and SPACE register  is

calculated using the following formulae:

MARK AFSK TONE CONSTANT  = MARK  TONE FREQUENCY * 4

SPACE AFSK TONE CONSTANT = SPACE TONE FREQUENCY * 4

For  example, to transmit a MARK tone of 2125 Hz and a SPACE tone of 2295  Hz,

the  MARK constant is 8,500 and the SPACE constant is 9,180.  The lowest  tone

constant is 1,200 for 300 Hz, and the highest is 12,000 for 3000 Hz.

4.3.8  MARK Filter Synthesizer

The MARK filter synthesizer is illustrated in Figure A1812.  This  synthesizer

generates  the clock, MCLK (U19-19), used to set the center frequency  of  the

MARK  bandpass  filter  on  the modem board (J7-1).   The  operation  of  this

synthesizer  is  identical to the transmit tone synthesizer described  in  the

previous section, and the output of this synthesizer is 50 times the  selected

MARK frequency.

The  MARK filter constant is loaded into two octal D-type latches (U45,  U46).

The  output frequency of the MARK filter synthesizer is calculated  using  the

following formula:

MARK FILTER CONSTANT = MARK TONE FREQUENCY * 4

4.3.9  SPACE Filter Synthesizer

The SPACE filter synthesizer is illustrated in Figure A1813.  This synthesizer

generates  the clock, SCLK (U21-19), used to set the center frequency  of  the

SPACE  bandpass  filter  on the modem board (J7-3).   The  operation  of  this

synthesizer  is  identical to the transmit tone synthesizer described  in  the

previous section, and the output of this synthesizer is 50 times the  selected

SPACE frequency.

The SPACE filter constant is loaded into two octal D-type latches (U49,  U50).

The  output frequency of the SPACE filter synthesizer is calculated using  the

following formula:

SPACE FILTER CONSTANT = SPACE TONE FREQUENCY * 4

4.3.10  HIGH SPEED Filter Synthesizer

The  HIGH  SPEED  filter synthesizer is illustrated  in  Figure  A1814.   This

synthesizer  generates  the  clock, HSCLK (U23-19), used  to  set  the  center

frequency  of the HIGH SPEED bandpass filter on the modem board  (J7-7).   The

operation  of this synthesizer is identical to the transmit  tone  synthesizer

described in the previous section, except that  the output of this synthesizer

is set to the center frequency of the HIGH SPEED filter plus 10,000 Hz.

The  HIGH SPEED filter constant is loaded into two octal D-type latches  (U52,

U53).  The output frequency of the HIGH SPEED filter synthesizer is calculated

using the following formula:

HIGH SPEED FILTER CONSTANT = ( CENTER FREQUENCY + 10,000 ) / 12.5

4.3.11  LOW PASS Filter Oscillator

The  LOW PASS filter oscillator controls the corner frequency of the low  pass

filter  in  the detector of the modem board.  The programmable timer  (U1)  in

Figure  A1807 generates this clock signal, LPCLK (U1-13), and it is passed  to

the  filter  on the modem board (J7-5).  This oscillator is simply  a  16  bit

counter  with a clock reference frequency of 4.9152 MHz.  The 16 bit  constant

is loaded into the programmable timer.

For  a given data rate, the LOW PASS filter constant is calculated  using  the

following formula:

LOW PASS CONSTANT = ( 4,915,200 ) / ( 96 * DATA RATE )

This constant varies between 5,120 for 10 bits per second to 43 for 1200 bps.

4.3.12  Regeneration

Figure A1815 illustrates the serial channel controller (U66) used for  receive

data  regeneration  and  the  remote control port.   This  controller  has  an

internal baud rate clock that sets the data rate for both channels.

When  regeneration is active (REGEN = 1) asynchronous characters  received  on

the RXDB (U66-27) input of the serial controller are passed to the TXDB  (U66-

25)  output.   Regeneration  generates each  receive  character  removing  any

received  jitter  and  noise.  Note that regeneration is  only  possible  when

receiving asynchronous start-stop characters with character lengths of 5 to  8

data bits.

The   serial   controller  operates  in  an  interrupt  driven   mode   during

regeneration.  Any character received on the RSCB (U66-27) input causes  INT_L

(U66-5)  to go low signaling a data available interrupt.   The  microprocessor

controller reads the serial controller to clear this interrupt flag.

Note  that regeneration can only work properly when the data presented to  the

RXDB  (U66-27)  input has polarity set such that a MARK equals  TTL  high  and

SPACE  equals TTL low.  This should be the case when the MARK tone is set  for the  correct tone and the receive data normal/reverse, RXDNR (U57-1), are  set

correctly  for the desired signal.  The regenerated signal on  RXDFR  (U63-11)

will have MARK high and SPACE low independent of the RXDNR setting.

Whether  regeneration is enabled or not, a data rate clock equal to  16  times

the  baud  rate  is  generated on the TXCLK  (U66-26)  output  of  the  serial

controller.   This  clock  provides  a time reference  signal  for  the  clock

recovery  circuit described in the next section.  The frequency of this  clock

is set by the BAUD RATE setting on the front panel; it is set to 16 times that

rate.

4.3.13  Remote Control Port

The dual channel serial controller (U66) channel A is connected to the  Remote

Control port on the ST-8000A.  In Figure A1815, note that the polarity of  the

data  on  this port may be set to RS232 or MIL188 levels with the  jumper  J5.

When  in  the RS232 position, J5SW low, the port has normal  RS-232  MARK  and

SPACE control levels.

The  remote  port REMRXD is converted to TTL level (U59-6) and passed  to  the

RXDA  input  on  the  serial controller (U66-13)  after  passing  through  the

RS232/MIL188  polarity gate.  The ST-8000A may read the Clear to Send,  REMCTS

(U59-8), to control output flow control.

The  remote  port  REMTXD signal comes from the TXDB (U66-15)  output  of  the

serial  controller and the RS232/MIL188 polarity gate.  The REMTXD  signal  is

combined  with the RTSA (U66-17) serial controller output so that this  signal

is  held in the MARK hold state when RTSA is low.  Note that the  RTSA  output

drives  the REMRTS (U62-8) signal on the remote control port.  In addition,  a

REMDTR (U62-11) and REMCTS (U62-6) signals are provided.

The  remote  port is configured as a mult-drop output where up to  8  ST-8000A

remote  port  transmit  data outputs, REMTXD, may be  connected  in  parallel.

Diodes D2 and D3 in Figure A1815 actively pull REMTXD to MARK and SPACE levels

when RTSA (U66-17) is low or enabled, and allow the REMTXD to float when  RTSA

is  high.  Thus, the output data line is connected only when this unit  has  a

response  to  transmit.  The zener diodes D1 and D4 limit  the  REMTXD  output

voltage  swing  to  MIL188 levels.  To improve noise immunity  on  the  common

transmit  data output line, REMTXD, one and only one ST-8000A in a group of  2

or more units should have jumper J3 set to connect the resistor to the  output

line.   This  resistor defaults the line to a MARK or negative  voltage  level

when no unit is actively sending.

4.3.14  Synchronous Data Clocks

Figure  A1816  illustrates the receive data clock,  RXCLK  (U60-11),  recovery

circuit for serial data received by the modem board.  Basically, this  circuit

synchronizes an internal clock to the edges in the serial receive data,  RXDTR

(U57-2), from the modem card.

Changes in the receive data signal generate pulses (U57-11) that reset a 4 bit

counter  (U58) each time a change occurs.  This counter is driven by  a  clock

running at 16 times the baud rate, TXCLKB (U58-1), and the output of the  most significant bit of the counter (U58-6) will change after 8 TXCLKB cycles.   If

this  clock is properly set, the clock transition occurs at the center of  the

data  bit  and  a correct receive data recovered  clock,  RXCLK  (U60-11),  is

generated.   Jumper  J4 selects either normal or inverted clock for  RS232  or

MIL188 receive data.

A  D-type  latch  is driven by the recovered clock  (U55-3)  to  re-clock  the

receive data (U55-5).  In some cases, re-clocking the data will reduce receive

data  jitter and improve system performance.  Note that the receive  data  re-

clocking latch will work with either synchronous or asynchronous receive  data

since  it  does  not depend on a start or stop bit,  nor  does  data  polarity

matter.

The  baud  rate  clock TXCLKB (U58-1) is simply divided by 16 in  a  four  bit

counter (U58) to generate the TXCLK (U60-3) synchronous transmit clock.  Since

the  transmit data input, TXDIN (U59-1), directly drives the  tone  generator,

there  is no need to change this clock phase for RS232 and MIL188  data.   The

ST-8000A transmits correctly in either case.